LAB EXPERIMENTS

ECA1408-EMBEDDED SYSTEMS

Exp-24:

Code:

#include <systemc.h>

SC_MODULE(Design) {

sc_in<bool> clk;

SC_CTOR(Design) {

SC_THREAD(task1);

SC_THREAD(task2);

}

void task1() {

for (int i = 1; i <= 3; i++) {

wait(clk.posedge_event());

cout << sc_time_stamp()

<< " Task 1: " << i << endl;

}

}

void task2() {

for (int i = 1; i <= 3; i++) {

wait(clk.posedge_event());

cout << sc_time_stamp()

<< " Task 2: " << i << endl;

}

}

};

int sc_main(int argc, char* argv[]) {

sc_clock clk("clk", 10, SC_NS);

Design d("d");

d.clk(clk);

sc_start(40, SC_NS);

return 0;

}

Result:

Exp-25:

Code:

#include <systemc.h>

struct Transaction {

int address;

int data;

};

SC_MODULE(Receiver) {

void receive(Transaction t) {

cout << "Address: " << t.address << endl;

cout << "Data: " << t.data << endl;

}

SC_CTOR(Receiver) {}

};

SC_MODULE(Sender) {

Receiver* receiver;

SC_CTOR(Sender) {

SC_THREAD(send);

}

void send() {

Transaction t;

t.address = 100;

t.data = 250;

wait(5, SC_NS);

cout << "Sending transaction" << endl;

receiver->receive(t);

}

};

int sc_main(int argc, char* argv[]) {

Receiver r("receiver");

Sender s("sender");

s.receiver = &r;

sc_start(10, SC_NS);

return 0;

}

Result:

Exp-26:

Code:

#include <systemc.h>

SC_MODULE(ProducerConsumer) {

sc_fifo<int> fifo;

SC_CTOR(ProducerConsumer) : fifo("fifo", 2) {

SC_THREAD(producer);

SC_THREAD(consumer);

}

void producer() {

for (int i = 1; i <= 5; i++) {

fifo.write(i);

cout << "Produced: " << i << endl;

wait(2, SC_NS);

}

}

void consumer() {

for (int i = 1; i <= 5; i++) {

int data = fifo.read();

cout << "Consumed: " << data << endl;

wait(3, SC_NS);

}

}

};

int sc_main(int argc, char* argv[]) {

ProducerConsumer pc("pc");

sc_start(30, SC_NS);

return 0;

}

Result:

Exp-27:

Code:

#include <systemc.h>

SC_MODULE(SemaphoreDemo) {

sc_semaphore sem;

SC_CTOR(SemaphoreDemo) : sem("sem", 1) {

SC_THREAD(task1);

SC_THREAD(task2);

}

void task1() {

sem.wait();

cout << "Task 1 acquired resource" << endl;

wait(5, SC_NS);

cout << "Task 1 released resource" << endl;

sem.post();

}

void task2() {

wait(1, SC_NS);

sem.wait();

cout << "Task 2 acquired resource" << endl;

wait(2, SC_NS);

cout << "Task 2 released resource" << endl;

sem.post();

}

};

int sc_main(int argc, char* argv[]) {

SemaphoreDemo d("d");

sc_start(15, SC_NS);

return 0;

}

Result:

Exp-28:

Code:

#include <systemc.h>

SC_MODULE(CountingDemo) {

sc_semaphore sem;

SC_CTOR(CountingDemo) : sem("sem", 2) {

SC_THREAD(task1);

SC_THREAD(task2);

SC_THREAD(task3);

}

void task(int id) {

sem.wait();

cout << "Task " << id << " acquired resource" << endl;

wait(4, SC_NS);

cout << "Task " << id << " released resource" << endl;

sem.post();

}

void task1() { task(1); }

void task2() { task(2); }

void task3() { task(3); }

};

int sc_main(int argc, char* argv[]) {

CountingDemo d("d");

sc_start(15, SC_NS);

return 0;

}

Result:

Exp-29:

Code:

#include <systemc.h>

SC_MODULE(InterruptDemo) {

bool high_pending, low_pending;

SC_CTOR(InterruptDemo) {

high_pending = false;

low_pending = false;

SC_THREAD(run);

}

void run() {

wait(5, SC_NS);

low_pending = true;

high_pending = true;

cout << "Interrupts received" << endl;

if (high_pending) {

cout << "High-priority interrupt serviced" << endl;

high_pending = false;

}

if (low_pending) {

cout << "Low-priority interrupt serviced" << endl;

low_pending = false;

}

}

};

int sc_main(int argc, char* argv[]) {

InterruptDemo d("d");

sc_start(10, SC_NS);

return 0;

}

Result:

Exp-30:

Code:

#include <systemc.h>

SC_MODULE(RTOSDemo) {

SC_CTOR(RTOSDemo) {

SC_THREAD(scheduler);

}

void scheduler() {

cout << "RTOS Scheduler Started" << endl;

wait(1, SC_NS);

cout << "High Priority Task Executed" << endl;

wait(1, SC_NS);

cout << "Medium Priority Task Executed" << endl;

wait(1, SC_NS);

cout << "Low Priority Task Executed" << endl;

cout << "Scheduling Completed" << endl;

}

};

int sc_main(int argc, char* argv[]) {

RTOSDemo d("d");

sc_start(5, SC_NS);

return 0;

}

Result: